# 1  Long-term adaptation pathways for Venice and its

# 2  lagoon under sea-level rise

3  Piero Lionello1,*, Valeria Di Fant2,3, Ulysse Pasquier4, Luigi Tosi5, Gone´ri Le Cozannet10,

4,15	6	7	8,16

## 4  Robert J. Nicholls	, Wolfgang Cramer , Roger Cremades , Carlo Giupponi	, Jochen

9	11	12	13	14,17

## 5  Hinkel , Adriano Sfriso

, Pietro Teatini

2,3

, Athanasios T. Vafeidis

, Georg Umgiesser	,

6  and Marjolijn Haasnoot

7  1Department of Biological and Environmental Sciences and Technologies - DiSTeBA, University of Salento, Lecce

8  73100, Italy

9  2Department of Climate Adaptation and Disaster Risk Management, Deltares, Delft 2629 HV, The Netherlands

10  3Department of Physical Geography, Faculty of Geosciences, Utrecht University, Utrecht 3584 CB, The Netherlands

11  4Tyndall Centre for Climate Change Research, University of East Anglia, Norwich NR4 7TJ, UK

12  5Institute of Geosciences and Earth Resources - CNR-IGG, National Research Council, Padova 35127, Italy

13  6The Mediterranean Institute of Biodiversity and Marine and 21 22 Continental Ecology IMBE – UMR CNRS 7263 /

14  IRD 237 / Aix Marseille Universite´ / Avignon Universite´, Marseille 13397, France

15  7Sustainability Research Institute, University of Leeds, Leeds LS2 9JT, UK

16  8Department of Economics, Ca’ Foscari University of Venice, Venezia 30121, Italy

17  9Department of Adaptation and Social Learning Global Climate Forum e.V. (GCF), Berlin 10178, Germany

18  10Coastal Risks and Climate Change Unit, Risks Department, BRGM – French Geological Survey, Orle´ans 45000,

19  France

20  11Department of Environmental Sciences, Informatics and Statistics, Ca’ Foscari University of Venice, Mestre

21  (Venice) 30172, Italy

22  12Department of Civil, Environmental and Architectural Engineering - DICEA, University of Padova, Padova 35131,

23  Italy

24  13Institute of Geography, Christian-Albrechts University Kiel, Kiel 24106, Germany

25  14Institute of Marine Sciences - ISMAR, National Research Council - CNR, Venezia 30122, Italy

26  15School of Engineering, University of Southampton, Southampton SO17 1BJ, United Kingdom

27  16Adapt@Ve Programme, Fondazione Eni Enrico Mattei - FEEM, Venezia 30124, Italy

28  17Klaipeda University - KU, Institute of Marine Research - JTU, Klaipeda 92294, Lithuania

29  *piero.lionello@unisalento.it

30  ABSTRACT

The substantial risks posed to Venice and its lagoon by ongoing and projected sea-level rise (SLR) require unprecedented long-term adaptation strategies. We map the evolution of development pathways and the progressive shrinking of the solution space as SLR advances, identifying adaptation tipping points and analysing the relative pros and cons of alternative measures. The analysis highlights trade-offs among environmental quality, heritage preservation, social well-being and relevant Sustainable Development Goals, and costs increasing with SLR. With present insufficient greenhouse gas mitigation policies, the current

31  open lagoon strategy, with mobile barriers and multiple accommodation measures, is likely to encounter hard limits within the current century. Follow-up strategies include ring-dikes isolating the city from the rest of the lagoon, or a closed lagoon with permanent coastal dams, each preserving different combinations of values while entailing major ecological and socio-cultural transitions. Under extreme SLR, relocation of monuments to suitable inland areas and abandonment would be the only remaining strategy, which might become unavoidable in the 22nd century under current climate policies and an Antarctic ice-sheet collapse. Rapid mitigation could still avoid the most disruptive long-term outcomes.

32  Introduction

33  Venice and its lagoon are an iconic environment, with outstanding and multiple cultural, monumental and environmental values,

34  which in 1987 led to their inclusion among the UNESCO world heritage sites as “an inseparable whole”1,2. This lagoon,

35  covering about 550km2, is the largest in the Mediterranean. It hosts important biodiversity, aquaculture activities and provides

36  substantial ecosystem services. These outstanding values, the huge flow of tourists (above 22M visitors per year3 with a

37  turnover worth about 2-3Ge per year), the presence of a major port, and population decline from 170,000 in the early 1950’s to

38  less than 50,000 in 20244 make Venice a sustainability hotspot.

39	The preservation of Venice has been a growing concern, as it has been increasingly flooded during the last 150 years, with

40  18 out of 28 extreme events (flooding of more than 60% of the city) in the last 23 years5. This is due to relative sea-level

41  rise (SLR)6, which has until now been caused in approximately equal parts by global SLR and local land subsidence due

42  to natural processes and anthropogenic activities7,8. Since 2022, a system of mobile barriers (MoSE, Modulo Sperimentale

43  Elettromeccanico9,10) has prevented extensive floods. MoSE closes the three lagoon inlets during storm surges and lies on the

44  bottom under normal conditions, thus allowing navigation and water exchange with the open sea. The process leading to MoSE

45  was triggered by the dramatic 1966 flood11 and initiated in 1973, when the Italian government established a legal framework

46  for safeguarding Venice. MoSE has demonstrated its capability to protect the city, but the increasing number of closures due to

47  future SLR will jeopardize its viability and have serious impacts on the lagoon ecosystem7,12–14.

48	SLR in the North Adriatic Sea is expected to closely follow the global SLR, posing unprecedented adaptation challenges, as

49  in 2020 more than 50% of Venice lies 0.80-1.20 m above the mean sea level (MSL), with a tidal range of about 1 m. By 2100,

50  compared to beginning of the century, the SLR in Venice is expected to reach 0.42m under a low (RCP2.6) and 0.81m (up to

51  1.80m cannot be excluded) under a very high (RCP8.5) emissions scenario8. This would lead to 15%, 70% and 98% of the city

52  centre being flooded daily at the astronomical tide maxima without defence structures. In 2300, global SLR will be in the range

53  0.3-3.1m and 1.7-6.8m in a low and very high emissions scenario, respectively, while a 16m rise cannot be ruled out15.

54	Since the establishment of Venice, centuries of multiple and diversified large scale human interventions have shaped an

55  original natural lagoon in the current strongly anthropic environment (Fig.1), such as in the 15th and 16th centuries river

56  diversions to prevent sediments from filling the lagoon16 and since the late 19th century the construction of embankments

57  to prevent the lagoon from expanding inland17. Consequently, the lagoon is surrounded by polders resulting from the large

58  subsidence that affected these former wetlands18), which are kept dry by a large network of pumping stations and drainage canals

59  (Fig.2). Ongoing and future SLR poses unprecedented risks to this unique environment. Given grossly insufficient international

60  efforts to reduce greenhouse gas emissions19 and the inertia of SLR, it is essential to contemplate radical transformations for

61  the city of Venice and its lagoon, considering the preservation of its monuments, lagoon ecosystem, traditions and cultures,

62  economic prosperity, well-being and safety of citizens, which in the absence of adaptation would ultimately be lost.

63	The objective of this study is to map the evolution of the adaptation solution space under rising sea levels and to identify how

64  physical, ecological, social and economic constraints progressively limit the viability of alternative strategies. This study does

65  not aim to perform a cost–benefit analysis, which is neither feasible nor appropriate given the irreplaceable nature of Venice’s

66  monumental and cultural heritage and the multidimensional values that cannot be (or would highly debatable) expressed in

67  monetary terms. Indicative cost estimates are used comparatively, to illustrate feasibility thresholds rather than to determine

68  optimal choices.

69	To provide the necessary counterfactual context, we qualitatively describe, for each strategy, the main values that would be

70  preserved relative to a no-action baseline. This framing clarifies that the analysis considers not only what adaptation costs, but

71  also what each pathway effectively saves, thereby situating the results within a decision-relevant perspective.

Source: Esri, Maxar, Earthstar Geographics, and the GIS User Community

Venice

Esri, TomTom, Garmin, FAO, NOAA, USG

a

b

72

Figure 1. a) Color bar shows the elevation of land areas around the Venice lagoon and of the lagoon islands superimposed to the background satellite picture. Values are with respect to the MSL at the end of the 20th century. The white line represents the jurisdictional boundary of the lagoon, which approximates its physical boundary except at the port. b) the Venice lagoon in the Mediterranean region. Maps produced using ArcGIS Pro 3.6.0 (Esri; https://www.esri.com/) with the Esri World Imagery basemap (credits: Esri, Maxar, Earthstar Geographics, and the GIS User Community).

73

Figure 2. Elevation profiles of the Venice lagoon boundaries outlining the elevation of primary defenses with respect to the present lagoon and sea level: dike (1) and seawall (2) on Pellestrina barrier island; dunes (3) and embankment (4) on Lido barrier island; dunes and embankments along the northern littoral (5); and embankments protecting the polders along the lagoon border (6, 7, 8). The locations of the transects are displayed on the map at the top of this panel. The panels report also the name of rivers and canals that are crossed by the transects. Data were downloaded from the MASE geoportal (https://gn.mase.gov.it/portale/en/home). Transect locations are shown on the overview map (top) and by the blue lines in the inset images; each panel also lists the names of the rivers and canals crossed by the transects. The overview map is based on a Sentinel-2 image (acquired 12 April 2024) accessed via Copernicus Browser (https://browser.dataspace.copernicus.eu/) on 8 January 2026 and processed by the European Space Agency (ESA). Insets use the Esri World Imagery basemap (credits: Esri, Maxar, Earthstar Geographics, and the GIS User Community). The figure was produced using ArcGIS Pro 3.6.0 (Esri; https://www.esri.com/) and Office 365 software (Microsoft; https://www.microsoft.com/).

74  Results

### 75  Adaptation strategies: suitabilities, limits and costs

76  Based on the available knowledge and technologies, we consider four strategies to address the risks posed by SLR to Venice

77  and its lagoon: open lagoon, ring-diking, closed lagoon and retreat, each of them combining multiple measures with different

78  SLR adaptation capabilities and costs (Figs.3, 4). For each strategy, we also indicate which social, economic, environmental

79  and cultural values would be preserved relative to a no-action baseline. This qualitative counterfactual provides the necessary

80  context for interpreting the indicative cost thresholds and clarifies that the comparison concerns not only what each option

81  costs, but also what it allows Venice to retain as sea level rises.

82

Figure 3. a) Illustrations of the strategies described in this article, with an additional line showing superlevees (part of the closed lagoon strategy).

83  Open Lagoon

84  The open lagoon strategy involves the present system of mobile barriers (MoSE), which closes the lagoon inlets during high

85  waters to reduce flood risk, while remaining otherwise open. Relative to a no-action baseline, the open-lagoon strategy initially

86  preserves the lagoon ecosystem, the monumental and cultural heritage, economic activities and residents’ well-being, though

87  its effectiveness rapidly diminishes as future relative SLR sets a limit to MoSE’s functional lifetime by increasing frequency

88  and duration of closures7. As closure frequency increases, the probability of malfunction or delayed operation rises, reducing

89  the level of protection afforded to monuments and the safety of residents14. addtitionally, this would limit port operations13,

90  degrade the lagoon ecosystems, and, in the case of multi-day long closures, require an appropriate sewage treatment system for

91  water quality and a large-scale pumping system to maintain lagoon levels below sea level. Altogether, an increased frequency

92  and length of closures will result in higher failure risk and additional costs (see section Methods for all cost estimates), which

93  are in the range of 0.25-1.1Ge for the large-scale pump system, 0.3Ge for the sewage treatment system, between 1.5 and

94  3.5Ge assuming a 0.75 m SLR (with respect to the beginning of the 21stcentury, here and the rest of the text) until 2100 for

95  MoSE operations and disrupted port activities during closures, growing to more than 0.1Ge per year in case of relative SLR

96  above 1m. Alternatively, the relocation of the port could be considered (with costs of about 4.1-4.5Ge). Adopting 2 or 6

97  months as an upper limit for annual closure duration would imply a relative SLR of about 0.50 or 0.75 m as the suitability

98  threshold for this strategy7.

99	Complementary accommodation measures can increase the effectiveness and extend the functional lifetime of the open

100  lagoon strategy.

101	Measures for large-scale accommodation include injecting seawater into the saline aquifers in the depth range 600 to 1000

102  m beneath the lagoon, to gradually raise its central portion by up to 0.30 m over a 10-year period, with even a 0.50 m uplift

103  possible if depths below 1000 m are considered20–22. Total costs of this measure are estimated in the range 0.3-0.4Ge and it

104  could extend the effectiveness of the open lagoon strategy to a SLR of 1.25 m.

105	Since the 1960s, accommodation has been extensively implemented in Venice by raising street pavements, adapting the

106  ground floor of individual buildings and placing removable door gates. This latter measure can be scaled up across the city by

107  installing structures protecting monumental areas (with costs higher than 1Ge), such as that implemented for the St. Mark’s

108  Basilica, but space availability and negative visual impacts limit it. De-composing and re-constructing in-situ individual

109  buildings on new higher foundations would have costs from 2 to 6 Ge depending on the fraction of the city centre and height

110  raised. These measures could extend the effectiveness of the open lagoon strategy to protect the main monuments, but not the

111  rest of the city, for a SLR of about 1.75 m. These accommodation measures do not, however, safeguard lagoon ecosystems,

112  traditions and cultural practices tied to lagoon use, safety and well-being of residents.

113  Ring dikings

114  Ring-dikes consist of dikes isolating the city centre and other settlements from the rest of the lagoon, which remains connected

115  to the open sea. Compared to no action, ring-diking preserves monuments, residential areas and most economic functions,

116  while altering the traditional landscape and weakening the physical and cultural connection between the city and its lagoon.

117  Ring-diking does not require the operation of the MoSE barriers and the lagoon remains continuously open to marine exchange.

118  Consequently, lagoon ecosystems would evolve under sea-level rise, climate change and other anthropogenic pressures,

119  but without the impacts of the engineering interventions of the open- or closedlagoon strategies. Ring-diking has to be

120  complemented with an efficient city sewage system and systems of pumps maintaining the internal water level below the

121  surrounding lagoon, with additional costs up to 0.65Ge. Construction costs are estimated from 0.5 to 4.5Ge depending on the

122  dike length and height23. Cost estimates are shown for ring dikes designed to withstand 2 m and 6 m of SLR, reflecting the fact

123  that this strategy can technically be upgraded to protection levels in this range, depending on available space and structure

124  heights. Impacts of ring-diking would be non-negligible on tourism, because of substantial landscape changes, but minimal

125  on the ecosystems of most of the lagoon. Emergency preparedness would be required in the case of failure of the defence

126  system. Propensity to live in Venice could be affected by lacking attitude to risk and loss of connection to the lagoon. Public

127  and commercial transportation, the port location and its organisation would need to be completely redesigned.

128  Closed lagoon

129  The closed lagoon strategy consists of transforming the lagoon into a coastal lake (similar to Marina Bay, Singapore and the

130  IJssel Lake, The Netherlands). This strategy preserves the city’s monumental fabric, residential areas, and tourism activities,

131  but sacrifices the natural lagoon ecosystem, thereby maintaining the urban cultural heritage while losing the environmental

132  context that historically shaped it. At difference with ring-diking it would not affect the city landscape and safety issues will be

133  similar to that of polders. It would require permanent dams across the lagoon inlets, raising of the barrier islands and of the

134  landward embankments separating the lagoon from the surrounding polders, with costs that could be as high as 0.54Ge, 4.8Ge

135  and 20Ge, respectively23. The technical limit of this strategy is determined by the maximum potential heights of dams and

136  barriers and it would probably occur later than in the ring-diking strategy because the defence structures will be located far

137  from the city without directly impacting it. The cost ranges corresponding to protection levels of 2 m and 10 m of SLR are

138  reported, indicating that this strategy can in principle be extended to very high SLR values. Ecosystem and biota losses would

139  be unavoidable. Actions would be needed to control water quality in the lagoon. The required system of permanently active

140  pumps would be much larger than for the ring-diking strategy. The closed lagoon strategy could be scaled up to protect against

141  even higher SLR by using a super levee approach, a structure with a width about 30 times its height, such as proposed for

142  some low-lying areas of Tokyo24,25 with costs likely exceeding 30Ge. The closed lagoon strategy would force port relocation

143  or provision of large-scale locks that would likely strongly affect its operation and profitability. Touristic activities and the

144  residential environment would be affected only partially. A complementary function of both the ring-dike and closed-lagoon

145  strategies is the possibility of managing pluvial and river inputs through dedicated pumping systems, which allow excess water

146  to be discharged outside of the protected areas (see section Pluvial and river flood risks).

147  Retreat

148  Planned relocation and abandonment by residents are two components of the retreat strategy. Compared to a no-action

149  counterfactual, this strategy preserves only selected monuments (if relocated), while the historical urban fabric, lagoon-based

150  culture, traditional lifestyles, and most economic activities would be irreversibly lost. Without active management, substantial

151  SLR would transform the present lagoon into a deeper marine environment, where non-indigenous species, warming and

152  more saline waters would establish a new ecological regime, fundamentally different from the historical lagoon that shaped

153  Venice’s cultural and natural heritage. Planned relocation consists in dismounting buildings and reassembling them in new

154  higher locations. This unprecedented and complex operation (applied at a small scale, e.g. for the Abu Simbel temple in

155  Egypt26) would not prevent the loss of the cultural, historical and monumental assets of the original settlement. Up-scaling

156  former projects produces costs estimates in the range 1-10Ge, but this could be increased ten times in the Venice context. The

157  relocated monuments could be visited by tourists and new residential areas built around them. The flooded remains would

158  progressively deteriorate and could be visited for a limited period by boat (and submarine). It would imply the loss of private

159  properties with eventual compensation costs of order from 5 to 6.5Ge27. Abandonment by residents could happen either

160  gradually, as residents and activities move away as the flooded areas increase in time, or abruptly, after catastrophic events

161  caused by the failure of the implemented defence system.

25	0.1  0 0.10 0.25 0.50 0.75  125	1.75	4.0	6.0	10.0	SLR

Movable barriers

Protection of individual buildiniliJ.S• De/recomposing individual buildings Soil up-lift

Sewage system Pumps

-	Lenght of measure, from implementation to tipping point

- Uncertainty in timing of implementation, or of tipping point Measure remains, but becomes part of another strategy

Relocation of port

◄

Ring-diking

- ♦NH++

�

�

�

�

Pumps Dams

Raising of barrier islands protections Raising of embankments

Pumps

Super levees

Relocation of monument Abandonment

Monuments Ecosystems Traditions & culture Economic prosperity Safety and wellbeing Monuments Ecosystems Traditions & culture Economic prosperity Safety and wellbeing

Monuments Ecosystems Traditions & culture Economic prosperity Safety and wellbeing Monuments Ecosystems Traditions & culture Economic prosperity

Safety and wellbeing

--------- -----

30.0

20.0

10.0

8.0

s

Q	6.0

4.0

Closed lagoon

=J Open lagoon

Super levees

Retreat

t,

()

0	2.0

1.0

0.75

0.50 	 Ring-dikings

0.25

-.25	-0.1  0 0.10 0.25 0.50 0.75  125	1.75	4.0	6.0	10.0	SLR

1900  1960 2000

162

Low (SSP1 RCP2.6)

Very high (SSP5-RCP8.5)

2100 I----,	2300

Figure 4. a) Measures involved in the four adaptation strategies and the SLR range for their functional lifetime (solid lines), with associated uncertainties (dashed lines) and continuation as components of other strategies (light solid line). Uncertainties are caused by decision processes, technical limits, dependencies on complementary measures. b) Suitability level of the four strategies for safeguarding monuments, lagoon ecosystem, traditions and cultures, economic prosperity, safety and well-being of residents. The thickness of the horizontal bars conveys qualitatively three relative levels (full, partial, nil) of suitability. For the open-lagoon strategy, bar thickness decreases with increasing SLR to represent its progressively diminishing effectiveness, while the light blue bars indicate the potential of accommodation measures. In case of retreat, absence of residents prevents the analysis of suitability for safety and well-being, while suitability for safeguarding ecosystems decreases with progressing SLR.

c) Broad range of indicative costs (y-axis) versus SLR (x-axis) combining lifetime information from panel with cost data from sections "Land uplift by seawater injection ..." and "Indicative cost estimates..". The limits of the boxes are obtained considering the minimum and maximum values of measures in panels a) and b), capturing both costs and lifetime uncertainties. The bottom annotation indicates the approximate relative SLR values (with respect to the end of the 20th century, blue arrow) and SLR likely (boxes) and very likely (whiskers) ranges under low (green) and very high (red) emission scenarios for 2100 and 2300. Dashed arrows refer to the case of a large Antarctic contribution to SLR. Lead times are not considered. they might be multidecadal as in the case of MoSE and other flood barriers28.

163  Adaptation Pathways

164  Adaptation pathways29 result from combinations of the strategies over time, with adaptation tipping points (ATP) marking the

165  switch from one strategy to another, when risk becomes too large (fig.5).

166	Nowadays, the system is in an incremental adaptation phase, during which the implemented mobile barriers defence

167  system (MoSE) in combination with accommodation can efficiently maintain an open lagoon. Our estimate is that, when

168  the relative SLR will be in the range 0.75-1.75m, an adaptation tipping point (ATP, where a strategy can no longer meet its

169  intended goals) to a transformational phase will necessarily occur, and Venice and its lagoon will necessarily shift to a condition

170  unprecedented in their long history with fundamental implications for the economy, ecosystems, residents and the universal

171  value of the UNESCO property. This transformation is unlikely earlier than the 2070’s, even in a very high emissions scenario,

172  and remains unlikely until the 22nd century in a low emissions scenario. However, the increased risk of pluvial and river

173  flooding during extreme sea-level events may reduce the operational lifetime of the open-lagoon strategy compared to the limits

174  imposed by SLR alone. The SLR upper limit of the open lagoon strategy, corresponding to the successful implementation of

175  all accommodation measures, depends on whether the socio-economic scenario will support them and on not fully explored

176  technologies (e.g. uplifting the ground with pumping). The ATP could be anticipated if the MoSE would show signs of stress

177  or malfunctioning or in the case of major failure leading to a catastrophic event during a high storm surge, which could even

178  trigger the abandonment of the lagoon. Extending the open lagoon strategy to its upper limit by implementing accommodation

179  options appears more expensive than the ring-diking or closed lagoon strategies.

180	When the ring-diking or the closed lagoon strategy would be adopted the system would enter into transformational

181  adaptation, whose first part is a resilient development opportunity phase, where the present city structure and functioning are

182  maintained, though the connection to the lagoon (ring-diking) or to the open sea (closed lagoon) would be disrupted. As the

183  upper limit of the ring-diking is lower than that of the closed lagoon strategy, the former could be adopted first and the latter

184  postponed to a higher SLR. Transitioning towards the transformational adaptation phase before reaching the upper limit of the

185  open lagoon strategy would be cheaper overall and it could be maintained up to a 6 to 10-meter SLR.

186	Retreat may be a plausible path before reaching the technical limits of other strategies, if the economic profits and socio-

187  cultural reasons supporting the maintenance of the city structure critically diminish. Otherwise, the first ATP towards the retreat

188  strategy is in the 4-10m SLR range. It would be determined by the capability of the dams to sustain the sea-level difference

189  (depending on technology and geomorphology) and by the social acceptance of living in an area where a dam failure might

190  not allow sufficient time to escape/evacuate. The decision to abandon by the population and the relocation of Venice would

191  have costs higher than any former strategy except the construction of a super levee. In this strategy, the costs of raising the

192  embankments separating the lagoon from the polders around Venice is not considered, as it needs to be analyzed in the context

193  of the adaptation strategy of the wider North Adriatic coast to SLR, which is beyond the scope of this study. Retreat before the

194  end of the 24th century is likely in a very high emissions scenario, but it is unlikely in a low emission scenario30,31.

Figure 5. Adaptation pathways and tipping points marking the switch from one strategy to another. This figures highlightes the decreasing and shifting solution space with SLR as the system evolves from incremental to transformational adaptation, consisting in climate resilient development or retreat. Annotation at the bottom indicates the approximate relative SLR values (with respect to the end of the 20th century, blue arrow) and SLR likely range (boxes) and very likely (whiskers) in a low (green) and very high (red) emission scenario in 2100 and 2300. The dashed arrows refer to the case of a large Antarctic contribution to SLR.

195  Discussion

196  This discussion is structured around five key themes. First, we outline the multiple goals at stake and the limits of monetary

197  valuation for cultural heritage. Second, we examine how strategy-dependent risks emerge from interactions with tourism, port

198  activities and ecological drivers. Third, we discuss how rising sea levels introduce adaptation tipping points that progressively

199  narrow the solution space. We then consider the long lead times required for major interventions, which make timely decisions

200  essential. Finally, we situate the Venice case within the broader context of global coastal challenges.

201	Multiple goals must be considered when evaluating SLR adaptation strategies for Venice (Fig.6): preservation of monuments,

202  lagoon ecosystems, living traditions and culture (goals linked to its UNESCO world heritage status and Sustainable Development

203  Goals (SDGs) 11, 14, 15), and safeguarding economic prosperity, safety and well-being of residents (linked to SDGs 3, 8 and

204  9).

205	The four proposed strategies differ in the values that they can safeguard as SLR progresses. The open-lagoon strategy

206  preserves all major values in the near term, but its capacity declines as closure frequency increases. Adding accommodation

207  measures extends the protection of monuments and urban functions, but does not prevent the progressive ecological degradation

208  associated with more frequent MoSE operations. Ring-diking preserves monuments, residential areas and most economic

209  activities for substantially higher SLR, while leaving the lagoon ecosystem largely free to evolve, but at the cost of weakening

210  the cultural and physical connection between the city and its lagoon and introducing residual safety risks linked to dike

211  failure. The closed-lagoon strategy maximizes protection of the monumental fabric and urban life at the expense of the lagoon

212  ecosystem, which would be irreversibly transformed. Finally, retreat safeguards only selected monuments, with irreversible

213  loss of the urban fabric, lagoon-based culture and present-day economic functions. Ultimately, the choice among adaptation

214  strategies depends on the priority assigned to different values and on the resources available, notwithstanding that losses will be

215  unavoidable under high SLR.

216	Protecting Venice’s economic system (its real estate, infrastructure, port activity and tourism) would provide an economic

217  rationale for even the most costly strategies, as the potential losses of a no-action future substantially exceed their indicative

218  costs. In contrast, preserving the lagoon ecosystem, while environmentally beneficial, contributes only marginally to economic

219  comparison. Citizens’ well-being has measurable economic value, but it remains lower than the cost of a full-scale defence

220  once SLR exceeds roughly 2 meters.

221	Methods for monetary valuation of non-market goods, including cultural heritage, can be applied, but they capture only a

222  single component of value32. Monetary valuation can inform decisions when the expected changes in assets are marginal. But

223  the potential loss of Venice is, by no measure, a marginal loss. While society must still assign a finite, though possibly very

224  large "willingness-to-pay" to protect heritage, cultural values transcend monetary representation, making the integration of

225  cultural and economic assessments indispensable for policy decisions. A unique historical asset such as the city of Venice is

226  ultimately priceless: no monetary estimate can replace the cultural meaning and collective memory embedded in the city. This

227  universal and irreplaceable value provides the fundamental motivation for maintaining protection for as long as technically

228  feasible.

229	Risks depend on the adopted strategy and its interactions with other environmental drivers including tourism, port activities,

230  and non-indigenous species. These drivers can amplify the impacts of SLR, making strategy-dependent risks a central element

231  of long-term planning.

232

233

Figure 6. Goals, adaptation strategies and drivers. Symbols show whether strategies achieve (YES/NO/Partially) each goal and drivers increase (YES/NO/Mixed) the risk of missing them. Open-lagoon effectiveness declines with SLR.

234	Frequent closures of the mobile barriers would affect water temperature and salinity33, potentially reverse the ongo-

235  ing decrease of pollution and eutrophication34–36 and impact the lagoon morphology by decreasing the import of marine

236  sediments37,38. Ecosystem risks would increase enormously under the closed lagoon strategy. Further risks are caused by

237  ongoing and future warming33,39,40, combined with mixed positive-negative effects of non-indigenous species on ecosystem

238  services40–42.

239	Living tradition and culture are threatened by accommodation measures that disrupt the city’s internal connectivity and by

240  strategies that alter the link between Venice and its lagoon, which would be completely lost under retreat. Further risks are

241  posed by non-indigenous species increasingly impacting artisanal fisheries43,44.

242	The safety of residents and visitors depends critically on the reliability of the current mobile barriers and of other defense

243  systems and it will be threatened by insufficient funding, maintenance gaps or technical malfunctions. The health of residents

244  would be at risk if measures to guarantee good water quality in and around the city are not thoroughly implemented.

245	All long-term protection strategies also carry the possibility of major structural failures. This aspect is particularly critical

246  for the ring-diking strategy because of the immediate proximity between the dikes and the historic city. A failure of even a single

247  segment during high sea levels would result in rapid inundation of the city centre, potentially with several metres of water depth

248  and very limited time for emergency response. Although such failure modes lie beyond the scope of the present analysis, their

249  existence underscores the need to complement long-term planning with robust risk-management and emergency-preparedness

250  considerations.

251	Tourism and port activities are major contributors to local economic prosperity. While tourism helps sustain the city’s current

252  service infrastructure and contributes indirectly to the preservation of its monumental heritage, it can also reduce residents’

253  quality of life, affect local traditions and culture45 and increase pollution46. Tourism is broadly compatible with the open

254  lagoon, closed lagoon, ring-diking strategies, which would maintain the existing visitor services. However, accommodation

255  measures could affect tourism if the usable urban areas or the internal connectivity of the city decrease. Retreat strategies would

256  imply a complete reorganisation of tourism and presumably major reduction of flows.

257	The future of the port of Venice will be strongly impacted by changing global commercial patterns (with eventually less coal

258  and oil, more multimodal container shipping, and a decrease of large cruise ships) associated with climate change mitigation

259  policies47–50 and competition with other large ports in the North Adriatic13. Local adaptive actions include changes in port

260  operation or quite possibly relocation of the port outside the lagoon.

261	Under a high-emission scenario and assuming no additional accommodation measures, the present open-lagoon strategy

262  is likely to become inadequate before the end of the century as the frequency and duration of closures continue to increase.

263  Because large-scale interventions, such as permanent barriers, ring dikes or major structural reinforcements, require lead times

264  of 30–50 years, the effective window for initiating a transition to alternative strategies is already approaching. If procedures and

265  institutional processes similar to those that led to the MoSE are followed, timely planning will be essential to ensure that any

266  new strategy becomes operational before the current one reaches its functional limits.

267	Pathways include ATPs, which are associated with large societal and environmental consequences, the commitment of

268  huge budgets (whose source/availability is not obvious) and long (multidecadal) preparation and implementation timescales.

269  Decisions around ATPs are shaped by society’s capacity to absorb the required changes, including institutional readiness,

270  long-term financial commitments and the acceptability of trade-offs among competing goals. In extreme cases abandonment

271  might result from limited budget, declining cultural identity, shrinking population and catastrophic events.

272	High SLR would threaten permanent flooding of large densely populated and highly productive areas along the entire North

273  Adriatic coast (with 4,800 km2 of land below 2 m elevation) and all adaptation strategies for Venice and its lagoon need to be

274  embedded within a large-scale regional adaptation strategy rather than considered in isolation.

275	The adaptation solution space will progressively shrink with SLR as in many low-lying regions51. For Venice this contraction

276  implies a transition from the present open lagoon strategy (incremental adaptation) to the ring-diking or closed lagoon (the

277  opportunity window for resilient, transformational adaptation) and ultimately to relocation-abandonment during the irreversible

278  retreat phase under very high SLR. The timing of these transitions will depend on the rate of SLR. Abandonment of the open

279  lagoon strategy appears unavoidable on multi-centennial time scales. While it could be postponed to the 22nd century under a

280  low emission scenario, it might be required within the end of the 21st century under a very high emission scenario. Mapping the

281  progressive contraction of the solution space therefore informs decision-makers not about which option is optimal in economic

282  terms, but about which values can still be preserved and for how long as sea level continues to rise.

283	Venice exemplifies the challenges that many low-lying coastal areas will face due to SLR over the coming centuries52, as

284  illustrated by the Maldives53, the Netherlands54–56, and coastal cities57. Most populated low-lying areas are already engaging

285  in adaptation to manage inundation and flooding , yet current approaches will probably be able to work for limited time as sea

286  level continues to rise. Ultimately difficult and costly decisions are inevitable. The adaptation pathways approach illustrated for

287  Venice has global relevance and thinking through potential pathways is an important step towards practical and realistic coastal

288  adaptation given the long-term commitment to SLR and the uncertainty in its rate.

289	Venice has faced transformation and adaptation to a changing environment throughout a millennial history, but future

290  challenges are unprecedented in terms of pace of change and structural dimension. While the historical evolution of Venice

291  and its lagoon was largely dependent on local cultural and economic capacity, it now requires solutions relying on large-scale

292  cooperation and depending substantially on drivers acting beyond local control.

293	Taken together, our results show that adaptation pathways provide a decision-relevant framework to clarify which long-term

294  choices Venice will face as sea level continues to rise. Each feasible strategy preserves a distinct configuration of economic,

295  environmental, social and cultural values, and the narrowing of the solution space with rising SLR highlights the need for early

296  judgement on which of these values future communities aim to retain. For decision makers, the pathways approach does not

297  prescribe an optimal option; rather, it makes explicit the trade-offs and the adaptation tipping points at which transitions become

298  unavoidable. This enables choices to be made with full awareness of the gains and losses across competing interests, and of the

299  lead times required for large-scale interventions. More broadly, Venice illustrates how pathways can guide robust long-term

300  coastal planning under deep uncertainty, helping societies align present decisions with the values they wish to preserve in the

301  centuries ahead.

302  Methods

### 303  Estimates of sea-level rise and land elevation

304  The SLR values in figure 3 and in the text are based on state of the art projections assessed in the 6th Assessment Report of the

305  IPCC15. Values specific for Venice can be extracted from public repositories providing consistent values with minor differences

306  that are not relevant for this study8,58–61.

307	While these data approximately consider land subsidence in the historical city centre, where values in the last 50 years,

308  after the regulation of groundwater use, are between 1 and 2 mm year−162, subsidence increases across the lagoon up to 4 to

309  6 mm/year in poorly consolidated salt marshes38,63. This analysis assumes that the effective regulation of groundwater use

310  continues. Hence, land subsidence is due to long term natural components largely due to the consolidation of the shallow

311  subsurface sediments, i.e. the Holocene depositional units, and its variability reflects the geological setting of the lagoon64.

312  Extrapolating to the next centuries the current rates of about 1-2 mm/year for the historic centre and 3-4 mm/year for the lagoon

313  is plausible, but anthropogenic activities (e.g. construction of dikes or dams) could locally affect it. Therefore, for low emission

314  scenarios, as long as the SLR velocity is in the same order of magnitude as the land subsidence, the relative SLR footprint in

315  Venice and its lagoon will be heterogeneous38 with substantial implications for the preservation of the lagoon ecosystems and

316  of monuments.

317	Timing of global SLR milestones up to 5 m until 2300 are accessible through the same website58 and are considered

318  adequate in this study for the long term estimates based on evidence that past (neglecting land subsidence interannual variability)

319  and future long term trends in Venice are very similar to global values7,65,66. Among other data used for this study, the IPCC

320  AR6 WG1 chapter 915 reports that global mean sea level will rise between 0.3 and 3.1 m and between 1.7 and 6.8 under

321  SSP1-2.6 and SSP5-8.5 by 2300, respectively, and in the latter case up to 16 m considering possible contributions from marine

322  ice cliff instability. Further studies, based on different methods and published after the 6th IPCC Assessment Report have

323  revisited low-likelyhood-high impact scenarios to 2300 without changing significantly these values30,31.

324	The map shown in Fig.1 is a digital terrain model (DTM) provided by the Veneto Region (https://idt2.regione.veneto.it/idt/downloader/down

325  which is intended to illustrate the general morphological setting of the area around Venice. The DTM is based on surveys

326  carried out in the 1990s and the data refer to the Italian national geodetic network, which in turn refers to the MSL determined

327  in the 1940s. Actually, land subsidence and SLR have led to a further reduction in ground elevation. Land subsidence, acting

328  with a large spatial variability ranges from negligible values up to 1 cm/year64,67, has contributed up to 20 cm in some areas,

329  while the SLR implies a uniform negative decrease of 1.2 mm/year8, i.e. about 10 cm compared to the 1940s. Practically these

330  two sources imply a negative systematic error with a magnitude of about 25-30 cm, which is too small to be relevant in this

331  analysis.

332	The fraction of historical centre flooded as a function of the sea level are computed using the data provided by Insula spa

333  (http://smu.insula.it/index.html).

### 334  Land uplift by seawater injection: values, technical details and costs

335  Fluid injection into the subsurface is used worldwide for multiple purposes (managed aquifer recharge, enhanced oil recovery,

336  geologic sequestration of CO2 and wastewaters, and underground gas such as CH4, H2 storage) and it is always accompanied

337  by an uplift of the land surface above the injection facilities. State-of-the-art 3D hydro-geomechanical modelling shows that

338  using 12 wells located on a 10-km diameter circle around the Venice city centre to inject seawater into the deep aquifer system

339  below the lagoon between 600 and 1000 m depth can produce an uplift of approximately 25-30 cm21. With the planned well

340  locations, the area benefiting by the uplift encompasses the Venice historic centre, Murano, and other islands around Venice.

341  A proper management of the injection rates, combined with the large depth of the targeted aquifers, would ensure a uniform

342  uplift, with displacement gradients that are substantially smaller than those caused by land subsidence over the past century,

343  precluding risks for building instability20. The maximum overpressure would occur well below the fracturing threshold. A

344  thick clay unit revealed by seismic surveys and borehole lithostratigraphy, will preclude any propagation of the saline water

345  toward the freshwater aquifers located above 350 m depth. Costs, initially estimated to be 0.08Ge for construction of the wells

346  and 0.1Ge for operating them for 10 years68, have now risen to a total in the range 0.3-0.4Ge considering the increased costs

347  of the drilling, equipment, and energy costs over the last 10 to 15 years. With the present market prices, annual maintenance

348  and operating costs, including electricity, seawater treatment, and personnel is estimated in 0.02Ge/year.

### 349  Extreme sea-level events and height of coastal defenses

350  In Fig.2 elevation profiles have been utilized to outline the primary coastal defenses located at the boundaries of the Venice

351  lagoon. Elevation datasets refer to the DTM with 1 and 2 m ground resolution resulting from LiDAR scanning on an

352  aerial platform acquired by the Ministry of the Environment and the Protection of Land and Sea under the Extraordinary

353  Environmental Detection Plan (PST-A). The datasets are available from the Ministry of the Environment and Energy Security

354  (https://gn.mase.gov.it/portale/en/home and https://sim.mase.gov.it/portalediaccesso/mappe/#/viewer/new).

355	Dunes in the areas without artificial protection have a level of about 9 m above MSL, although a detailed inspection reveals

356  the presence of small isolated gaps where the maximum elevation is between 2 and 3 m. The height of the current defenses

357  along the barrier islands is approximately 5 m. Currently, dykes and riverbanks approximately 4-meters high separate the lagoon

358  from the adjacent polders. Therefore, a head height of 5 m above mean sea level is sufficient to prevent wave overtopping

359  during major storms and damage to inhabited areas.

360	These values of the height of the coastal defenses are consistent with other estimates. The 100-year return level has

361  been estimated to be approximately 1.50 m69, but excluding the contribution of astronomical tides and of interannual to

362  seasonal sea-level variability. During major past events, the contribution of the interannual to seasonal sea-level variability

363  never exceeded 20cm. Considering a further addition of astronomical tide (maximum of about 50cm) confirms that 3 m are a

364  reasonable upper limit for sea-level maxima, which with a 60% addition to prevent wave overtopping would lead to a height of

365  5m.

366	The future increase of extremes during storms will be mostly produced by SLR70, with changes of storminess playing a

367  minor role71. The effect of SLR on astronomical tides will depend on the future large scale adaptation strategy72. Preserving the

368  present coastline of the Adriatic will lead to a decrease of the amplitude of astronomical tides, while allowing a free expansion

369  of the sea inland will increase the amplitude of the diurnal components. As a consequence, it has been estimated that a 2-meters

370  SLR might lead to a 5% increase or decrease of the amplitude extreme events72.

371	In conclusion, 3-meters high ring dikes inside the lagoon, where wave propagation from the sea is prevented by barrier

372  islands, and 5-meters high dams across the inlets and embankments along the barrier islands, both with respect to the mean sea

373  level, are expected to provide sufficient protection.

374	The height of embankments separating the lagoon from the surrounding polders would be different depending on whether

375  the closed lagoon or the ring-diking strategy is adopted. In the former case, the water level inside the lagoon will remain at its

376  present level and the increase of the embankments will not be required. In case of ring-diking the water level inside the lagoon

377  will follow the SLR and the height of the embankments should match that of the dikes. However, the protection of the polders

378  will require an overall revision of the coastal protection and hydraulic drainage system at regional level, beyond the adaptation

379  of the Venice lagoon to SLR, whose discussion is beyond the scope of this article.

Table 1. Total costs associated with the operation of the MoSE system. total duration of closures is based on literature7. Total costs include operation of the barriers and disruption of port activities.

SLR (cm)	yearly duration of closures (months)	Total cost (Ge/year)

| 50 | 2 | 0.015 - 0.031 |
| --- | --- | --- |
| 75 | 6 | 0.044 - 0.093 |
| 120 | 10.5 | 0.077 - 0.162 |

### 380  Pluvial and river flood risks

381  Although this study focuses on long-term sea-level rise, pluvial and riverine inputs represent an additional source of flooding

382  that becomes relevant whenever the lagoon—or parts of it—is temporarily (during MoSE closures) or permanently (under

Table 2. Total Costs of raising existing buildings. Unit costs (column 2) in UST/ft at 2010 prices are extracted from literature73 and converted to Ge 2024 prices in columns 3 (35% of buildings) and 4 (all buildings in the historical centre)

raise buildings by (m)	Unit cost (USD/ft2)	Total cost for 35% of buildings (Ge)	costs for all buildings (Ge)

| 0.6 | 88 | 1.01 | 2.83 |
| --- | --- | --- | --- |
| 1.2 | 91 | 1.05 | 2.93 |
| 1.8 | 96 | 1.10 | 3.09 |
| bottomrule |  |  |  |

Table 3. Ring dikes with indicative construction (top) and pumping costs (bottom). Each dike has a head of 3 m, for a total height obtained by adding head, SLR and a depth of 4 m around Venice’s historical centre, and 2 m in the other dikes. The large perimeter ring dike corresponds to a separate configuration that would enclose Venice’s historical centre as well as other developed areas. Different SLR conditions are considered only for construction costs, as they have a negligible effect on pumping (including energy and maintenance) costs. Pumping costs are computed to cover a 50 years duration.

CONSTRUCTION COSTS

| ring diking area | length (km) | area (km2) | present SL (Ge) | 2 m SLR (Ge) | 4 m SLR (Ge) | 6m SLR (Ge) |
| --- | --- | --- | --- | --- | --- | --- |
| Venice | 15.8 | 10.3 | 0.45-1.24 | 0.58-1.60 | 0.71-1.96 | 0.84-2.31 |
| Murano | 5.3 | 1.7 | 0.11-0.30 | 0.15-0.42 | 0.20-0.54 | 0.24-0.66 |
| Murano-Mazzorbo-Torcello | 8.9 | 3.1 | 0.18-0.50 | 0.25-0.70 | 0.33-0.90 | 0.40-1.10 |
| large | 36.2 | 99.3 | 0.74-2.04 | 1.04-2.85 | 1.33-3.66 | 1.63-4.48 |
| bottomrule |  |  |  |  |  |  |

| PUMPING (energy and maintenance) COSTS |  |  |
| --- | --- | --- |
| ring diking area | area | from present SL up to 6m SLR |
|  | (km2) | (Ge) |
| Venice | 10.3 | 0.02-0.07 |
| Murano | 1.7 | 0.003-0.01 |
| Murano-Mazzorbo-Torcello | 3.1 | 0.005-0.02 |
| large | 99.3 | 0.16-0.65 |
| bottomrule |  |  |

383  ring-dike or closed-lagoon strategies) isolated from the sea.

384	Recent observational analyses for Italy show suggestive evidence of increasing heavy rainfall intensity, despite spatial

385  heterogeneity in the trends74,75. Future projections indicate a widespread intensification of precipitation extremes across

386  northern and northeastern Italy under high-emission scenarios76.

387	Intense precipitation and river inflows can raise water levels inside a closed lagoon independently of the sea level.

388  Hydrodynamic simulations and observations indicate that rainfall over the lagoon surface and river discharge may increase

389  lagoon water levels by several centimetres during MoSE closures12,33,77. Comparable risks would arise under the closed-lagoon

390  strategy and, to a lesser extent, under ring-diking. In the latter case, river flooding would be irrelevant, and only precipitation

391  falling on the protected areas would need to be managed.

392	Installing pumping systems is an effective way to reduce pluvial and river-flood risks. For a fully closed lagoon, a first-order

393  engineering estimate indicates that managing precipitation and river inflows during extreme events requires a peak pumping

394  capacity of approximately 1,000 m3s−1 under present climate conditions. This estimate is obtained by summing the current

395  estimated river peak discharge78 and the contribution of the maximum daily precipitation from observational gridded data

396  in the area around the lagoon75. Assuming a 20% increase in precipitation extremes by the end of the century under a very

397  high-emissions scenario, the required peak pumping capacity would rise to roughly 1200 m3s−1.

398	Pumping requirements would be substantially lower for the ring-dike strategy because the contributing surface is much

399  smaller and river inputs are excluded. Incorporating these operational needs into future assessments would imply increasing

400  long-term pumping-system costs by a factor of 2 to 3 for both strategies.

Table 4. Indicative construction costs for dams at lagoon inlets, embankments along the barrier islands and the borders of the lagoon assuming a head of 5 m and SLR of 2, 6, 10 m. The costs are based on the total height, which is estimated adding depth, SLR and head

CONSTRUCTION COSTS

| inlet | depth (m) | length (m) | present SL | 2 m SLR (Ge) 6 m SLR (Ge) |  | 10 m SLR (Ge) |
| --- | --- | --- | --- | --- | --- | --- |
| Lido 1 | 8 | 420 | 0.02-0.06 | 0.03-0.07 | 0.03-0.09 | 0.04-0.11 |
| Lido 2 | 8 | 440 | 0.02-0.06 | 0.03-0.07 | 0.03-0.09 | 0.04-0.11 |
| Malamocco | 14 | 540 | 0.04-0.11 | 0.05-0.13 | 0.06-0.15 | 0.06-0.18 |
| Chioggia | 8 | 550 | 0.03-0.08 | 0.03-0.09 | 0.04-0.12 | 0.05-0.14 |
| Total bottomrule embankments |  |  | 0.12-0.32 | 0.13-0.37 | 0.16-0.45 | 0.20-0.54 |
| barrier islands |  |  | 0.58-1.60 | 0.83-2.22 | 1.3-3.53 | 1.75-4.81 |
| lagoon borders bottomrule |  |  | 2.45-6.75 | 3.44-9.45 | 5.40-14.9 | 7.36-20.02 |
| PUMPING (energy and maintenance) COSTS |  |  |  |  |  |  |
|  | area (km2) |  | from present SL up to 6m SLR (Ge) |  |  |  |
| whole lagoon bottomrule | 550 |  | 0.44-1.78 |  |  |  |

### 401  Indicative cost estimates of the strategies and of the corresponding measures

402  Indicative capital costs for the proposed measures and adaptation pathways are estimated using unit costs from previous

403  literature, international analogs and expert judgement (fig.7). This analysis is a first approximation, which has the high relevance

404  of being the first to attempt a systematic assessment of the order of magnitude of the broadest conceivable set of long term

405  adaptation strategies. Unless specified, all amounts presented here are adjusted for inflation to 2024 prices. Due to the city’s

406  historical and cultural importance, as well as its unique geography, costs in Venice tend to be much higher than comparable

407  experiences elsewhere. Our estimate is subjected to this systematic uncertainty and might be a systematic underestimate of

408  order up to 50%. While cost will be a determinant factor in the realization of individual strategies, it is important to note

409  that our analysis aims at providing indicative estimates rather than precise values mainly in order to compare the costs of the

410  different options. As a reference, all the estimated costs could be compared to the 6Ge total cost for the construction of the

411  MoSE system13.

412  Open lagoon

413  Maintaining the open lagoon includes the costs of operating MoSE and of the implementation of the pumping and sewage

414  systems required by frequent closures of the lagoon. The increase of MoSE costs with SLR are summarised in Table 1. They

415  are obtained adding operating costs (between 30 and 300 Ke per closure) to costs caused by disruption of ship traffic (average

416  215Ke per closure79, assuming daily durations and that prolonged multi-day closures are equivalent to the sum of individual

417  closures13. Total estimates would depend on the rate of SLR. As an order of magnitude estimate, one could assume costs

418  linearly increasing over 75 years until there is a 6 month annual duration of closures, with total costs in the range 1.5 to 3.5 Ge.

419	Costs (not considering maintenance) of pumping systems vary between 0.47 and 2 Me/m3/s and depend on factors such as

420  length of pipes, distance to source area, motor type and drainage area80. Assuming that the pumping system is designed to

421  respond to the maximum peak flow of the Venice lagoon tributaries and sub-basins (a reference value of 344 m3/s78 and to the

422  precipitation inside the lagoon (about 200 m3/s in case of a 50mm daily precipitation), the construction of the pumping system

423  for a lagoon-scale system would cost from 0.25 to 1.1Ge.

424	A wastewater treatment plant currently under construction for an area of 150,000 people in Trento in Northern Italy, is

425  estimated at a cost of 0.113Ge, with an expected operational cost of 1.3Me/year considering current costs in the region81.

426  These costs are roughly consistent with the upper limit of the estimated costs for conventional sewage systems (USD 770 per

427  inhabitant at 2010 prices82. Considering the complexity of creating a collection system in the historical centre, the unit costs

428  can be multiplied by at least a factor of two with respect to the cost sustained in Trento, reaching a cost in the order of 0.3Ge,

Figure 7. Indicative costs of the measures (in Ge). Each bar extends from the minimum to the maximum estimated cost. For the ring-diking and closed lagoon strategy, the costs corresponding to 2 and 6 m and to 2 and 10 m, respectively, are shown.

429  including operating it for 75 years. Dry-proofing individual buildings (e.g. protective shields, external impervious coating, etc.)

430  is already ongoing in Venice. Costs vary between USD9,000 and USD23,000/building (2016 prices) in developed countries80.

431  Applying this strategy to the 2740 buildings below an elevation of 1 m in Venice’s historical centre would thereby cost from

432  0.03Ge to 0.08Ge. Maintenance costs per year are estimated at 1-3% of the investment cost80. This is a low cost option

433  compared to other accommodation measures that would be needed to extend the lifetime of the open lagoon strategy.

434	In 2022, a glass barrier was installed to protect St. Mark’s Basilica. The 150m-long and 1.3m-high barrier is reported to

435  have cost 5.3Me83 and is equipped with a pumping system. Taking the same unit cost and height as the barrier protecting the

436  Basilica, deploying this measure to protect exposed (below 1m elevation) buildings of cultural significance would cost 1.1Ge

437  (excluding maintenance costs). In this calculation, 245 buildings are considered and it is assumed the glass barriers align with

438  respective building perimeters.

439	The reconstruction and raising of existing buildings is a much more expensive process. An estimate for building types

440  in Los Angeles73, provides unit costs per building footprint that highly dependent on the type of structure, with “masonry

441  structure with slab-on-grade" being the most fitting category to buildings found in Venice. Costs in Table 2 consider an average

442  building footprint of about 1363 m2 (calculated using OpenStreetMap features). An important caveat of these estimates is the

443  extremely soft subsurface layers, the complexity of structures and foundations in Venice, which would lead to much greater

444  costs than in other cities. It is therefore plausible to apply a factor 2 or larger to the costs in Table 2.

445

446

447

448

449

450

451

452

Ring-diking

Two different ring diking configurations are considered. The first comprises three ring dikes around (1) Venice’s historical centre and Giudecca, (2) Murano and (3) Burano-Mazzorbo-Torcello. The second configuration covers a much larger perimeter to enclose Venice as well as developed surrounding islands, connecting the dike to the Lido barrier island. The depth near Venice’s historical centre is assumed to be deeper than in the rest of the lagoon at 4 m compared to 2 m for the other dikes. A hydraulic head of 3 m is targeted for each dike.

Energy costs of pumping CPE would increase with SLR because of increasing difference of water level between the two sides of the dikes:

φ	6

CPE = c   ρgVH  ,	(1)

3.6 · 10

453  where ρ is water density (about 1030 Kg/m3), g is gravity (9.8 m/s2 ), V is the total annual precipitation (800mm), H (is

454  the level difference, m), φ the pump efficiency (0.7). Assuming a cost of 0.2e/kWh and a maintenance cost of 3% of the

455  capital cost (ignoring SLR related compounding factors such as salinity, corrosion and more frequent operations) shows that

456  the former ones are a minor fraction of the latter ones (in the range from 5% to 20%). Table 3 shows the dike and drainage

457  area characteristics and associated construction and pumping (including energy and maintenance) costs based on available

458  estimates23.

459  Closed lagoon

460  The permanent closure of the lagoon requires the construction of stationary barriers across the present lagoon inlets, the raising

461  of barrier islands to prevent encroachment and possibly of the whole system of canal banks around the lagoon separating it

462  from the surrounding polders.

463	The unit cost of sea dikes for Italy is estimated at 4.09-11.25 Me/km/m23 , adjusted to 2024 prices. Yearly maintenance

464  costs are assumed to be 3% of construction costs. Table 4 presents estimated costs of the permanent barriers with a 5-meter

465  hydraulic head for different SLR values.

466	A closed lagoon strategy might require elevating the banks separating the lagoon from surrounding polders either to avoid

467  “backdoor” flooding of the lagoon or flooding of the polders, depending on regional adaptation strategy. A 5m-high dike over

468  the 120 km length of the lagoon perimeter would cost up to 20Ge23 in case of a 10-meter SLR. This cost would be about 90%

469  lower if a 3-meter head and the present water level inside the lagoon are assumed. The cost for a stone embankment of the

470  same height stretching 28.5 km on the Lido, Pellestrina barrier islands and the Sottomarina peninsula to the South of the lagoon

471  can be estimated in a similar way in the range 0.83 – 4.81 Ge depending on SLR. Pumping costs of the closed lagoon strategy

472  (estimate in the range 0.44-1.78Ge) would be much higher than for the ring-diking strategy as the water level of the whole

473  should be kept fixed. The coastal offshore front of the lagoon could also be considered for a “super-levee”. An example has

474  been conceptualised in Tokyo, Japan25. This structure would be much wider (a width equal to 30 times its height) and therefore

475  significantly more expensive than a simple dike and reliant on available space. The cost of a super-levee of roughly the same

476  length (44 km) in Edogawa was estimated at 16.91Ge25.

477	The cost associated with the relocation of Venice’s port is derived from the unit cost of USD millions 16.6/km2/m50. This

478  value is based on costs reported for the Maasvlakte2 development in Rotterdam and includes land reclamation and construction

479  of infrastructure. The current Port of Venice covers an area of 20.45 km2. For a maximum depth of 13.5 m the cost of relocation

480  is estimated at 4.1Ge. For an additional 1.2 m depth, the cost increases to 4.47Ge. Costs for sewage treatment are to be

481  considered as part of the closed-lagoon strategy, as it was described for the open-lagoon strategy.

482  Relocation

483  The relocation of historical monuments is a costly and complex operation. The example of Abu Simbel in 1968 cost between

484  M$ 40 and M$ 70, which corresponds to 0.34Ge - 0.56Ge in 202426. This was a relatively small-scale operation compared to

485  the extent of relocation that would be required in Venice. Adopting the cost of Abu Simbel to the relocation of 3 to 30 buildings

486  would therefore result in a range from 1 to 10Ge. However, this is a very uncertain estimate, because of unknown technical

487  issues and dependence on the number of considered buildings.

488  Abandonment

489  Low direct costs can be assumed for the abandonment strategy, though the removal of environmentally damaging material (e.g.

490  oil) would need to be considered. However the relocation of residents would have relevant costs. An example is the scheme

491  offered by the Austria government (Relocation as adaptation to flooding in the Eferdinger Becken, Austria), which offered 80%

492  of their house value to people as compensation if they agreed to relocate from areas prone to floods. Considering market price a

493  100m2 apartment in Venice would have a cost of about 0.33Me, which would bring a compensation costs of 0.26Me per each

494  household. Considering 25,000 households in total would imply a total compensation cost of about 6.5Ge. If, alternatively, a

495  cost of 3 x GDP/capita (approximately 35Ke) per person moved is assumed27, the cost would be about 5.25Ge considering

496  50,000 inhabitants. Furthermore, abandonment would imply the cancellation of the tourism sector, which has presently an

497  annual turnover of approximately 1Ge84.

498  References

| 499 | 1. |
| --- | --- |
| 500 | 2. |
| 501 |  |
| 502 | 3. |
| 503 |  |
| 504 | 4. |
| 505 |  |
| 506 | 5. |
| 507 |  |
| 508 | 6. |
| 509 |  |
| 510 | 7. |
| 511 |  |
| 512 |  |
| 513 | 8. |
| 514 |  |
| 515 | 9. |
| 516 |  |
| 517 |  |
| 518 | 10. |
| 519 |  |
| 520 | 11. |
| 521 |  |
| 522 | 12. |
| 523 |  |
| 524 | 13. |
| 525 |  |
| 526 | 14. |
| 527 |  |
| 528 | 15. |
| 529 |  |
| 530 |  |
| 531 |  |
| 532 | 16. |
| 533 |  |
| 534 |  |

ICOMOS, I. C. o. M. & Sites. Insular Venice and its lagoon (1987). Available at https://whc.unesco.org/document/153362. UNESCO, S., UNITED NATIONS EDUCATIONAL & ORGANIZATION, C. Report sc-87/conf.oos/9 (1988). Available

at https://whc.unesco.org/document/904.

Bertocchi, D., Camatti, N., Giove, S. & van Der Borg, J. Venice and overtourism: Simulating sustainable development scenarios through a tourism carrying capacity model. Sustainability 12, 512 (2020).

di Venezia, C. popolazione residente (last updated 11/3/2025). Available at https://www.comune.venezia.it/it/content/ grafici-e-statistiche.

CPSM, C. d. V., Centro Previsioni e Segnalazioni Maree. Grafici e statistiche (last updated 14 february 2024). Available at https://www.comune.venezia.it/it/content/grafici-e-statistiche.

Lionello, P. et al. Severe marine storms in the northern adriatic: Characteristics and trends. Phys. Chem. Earth, Parts A/B/C 40-41, 93–105, DOI: https://doi.org/10.1016/j.pce.2010.10.002 (2012).

Lionello, P., Nicholls, R. J., Umgiesser, G. & Zanchettin, D. Venice flooding and sea level: past evolution, present issues, and future projections (introduction to the special issue). Nat. Hazards Earth Syst. Sci. 21, 2633–2641, DOI: 10.5194/nhess-21-2633-2021 (2021).

Zanchettin, D. et al. Sea-level rise in Venice: historic and future trends (review article). Nat. Hazards Earth Syst. Sci. 21, 2643–2678, DOI: 10.5194/nhess-21-2643-2021 (2021).

Colamussi, A.  Venice high water barriers:  Problems analysis and design approach.  Tech. Rep., ICCE 1992 local organising committee (1992). Accessed on 21 june 2025 at https://repository.tudelft.nl/file/File_ 0b02df37-be8d-42f7-a36c-78e2f04319ac?preview=1.

Bourdeau, P. et al. Report on the Mobile Gates Project for the Tidal Flow Regulation at the Venice Lagoon Inlets (Collegio di Esperti di Livello Internazionale, Brussels, Belgium, 1998).

De Zolt, S., Lionello, P., Nuhu, A. & Tomasin, A. The disastrous storm of 4 november 1966 on italy. Nat. Hazards Earth Syst. Sci. 6, 861–879, DOI: 10.5194/nhess-6-861-2006 (2006).

Umgiesser, G. The impact of operating the mobile barriers in Venice (mose) under climate change. J. for Nat. Conserv. 54, 125783, DOI: https://doi.org/10.1016/j.jnc.2019.125783 (2020).

Giupponi, C. et al. Boon and burden: economic performance and future perspectives of the Venice flood protection system.

Reg. environmental change 24, 44, DOI: doi.org/10.1007/s10113-024-02193-9 (2024).

Haigh, I. D. et al. Rapid acceleration in the number of closures of storm surge barriers in the future: A new tool for estimating barrier closures. Preprints DOI: 10.20944/preprints202410.2298.v1 (2024).

Fox-Kemper, B. et al. 2021: Ocean, cryosphere and sea level change. In Masson-Delmotte, V. et al. (eds.) Climate Change 2021: The Physical Science Basis. Contribution of Working Group I to the Sixth Assessment Report of the Intergovernmental Panel on Climate Change, 1211–1362 (Cambridge University Press, Cambridge, United Kingdom and New York, NY, USA, 2021).

Bondesan, A. & Furlanetto, P. Artificial fluvial diversions in the mainland of the lagoon of Venice during the 16th and 17th centuries inferred by historical cartography analysis. Géomorphologie: relief, processus, environnement 18, 175–200, DOI: doi.org/10.4000/geomorphologie.9815 (2012).

535 17. Tosi, L., Carbognin, L., Teatini, P., Rosselli, R. & Gasparetto-Stori, G. The ISES project subsidence monitoring of the

536	catchment basin south of the Venice lagoon (Italy). Land subsidence 2, 113–126 (2000).

537  18. Gambolati, G. et al. Peat land oxidation enhances subsidence in the Venice watershed. Eos, Transactions Am. Geophys.

538	Union 86, 217–220 (2005).

539  19. Programme, U. N. E. Emissions gap report 2024: No more hot air . . . please! with a massive gap between rhetoric and

540	reality, countries draft new climate commitments (2024-10).

541 20. Gambolati, G. et al. On the uniformity of anthropogenic Venice uplift. Terra Nova 21, 467–473 (2009).

542  21. Teatini, P., Castelletto, N., Ferronato, M., Gambolati, G. & Tosi, L. A new hydrogeologic model to predict anthropogenic

543	uplift of Venice. Water Resour. Res. 47 (2011).

544  22. Gambolati, G. & Teatini, P. Venice shall rise again: Engineered uplift of Venice through seawater injection (Elsevier,

545	London, 2013).

546  23. Nicholls, R., Hinkel, J., Lincke, D. & van der Pol, T. Global investment costs for coastal defense through the 21st century.

547	Project Report, World Bank (2019).

548  24. Esteban, M. et al. Adaptation to sea level rise: Learning from present examples of land subsidence. Ocean. & Coast.

549	Manag. 189, 104852 (2020).

550  25. Cao, A., Esteban, M. & Onuki, M. Public support for flood adaptation policy in tokyo lowland areas. Clim. Policy 24,

551	1275–1292 (2024).

552  26. Hill, A. C. ‘the battle for abu simbel’: Archaeology and postcolonial diplomacy in the UNESCO campaign for Nubia. J.

553	Contemp. Hist. 56, 502–521, DOI: doi.org/10.1177/0022009421997884 (2021).

554  27. Lincke, D. & Hinkel, J. Coastal migration due to 21st century sea-level rise. Earth’s Futur. 9, e2020EF001965, DOI:

555	https://doi.org/10.1029/2020EF001965 (2021).

556  28. Haasnoot, M. et al. Adaptation to uncertain sea-level rise; how uncertainty in antarctic mass-loss impacts the coastal

557	adaptation strategy of the netherlands. Environ. Res. Lett. 15, 034007, DOI: 10.1088/1748-9326/ab666c (2020).

558  29. Haasnoot, M., Di Fant, V., Kwakkel, J. & Lawrence, J. Lessons from a decade of adaptive pathways studies for climate

559	adaptation. Glob. Environ. Chang. 88, 102907 (2024).

560  30. van de Wal, R. S. W. et al. A high-end estimate of sea level rise for practitioners. Earth’s Futur. 10, e2022EF002751

561	(2022).

562  31. Bamber, J., Oppenheimer, M., Kopp, R., Aspinall, W. & Cooke, R. M. Ice sheet and climate processes driving the

563	uncertainty in projections of future sea level rise: Findings from a structured expert judgement approach. Earth’s Futur.

564	10, e2022EF002772 (2022).

565  32. Licciardi, G. & Amirtahmasebi, R. The economics of uniqueness: investing in historic city cores and cultural heritage

566	assets for sustainable development (World Bank Publications, 2012).

567  33. Ferrarin, C., Bonaldo, D., Bergamasco, A. & Ghezzo, M. Sea level and temperature extremes in a regulated lagoon of

568	Venice. Front. Clim. 5, 1330388 (2024).

569  34. Sfriso, A. et al. Long-term changes of the trophic status in transitional ecosystems of the northern adriatic sea, key

570	parameters and future expectations: The lagoon of Venice as a study case. Nat. Conserv. 34, 193–215 (2019).

571  35. Zonta, R., Cassin, D., Pini, R. & Dominik, J. Substantial decrease in contaminant concentrations in the sediments of the

572	Venice (italy) canal network in the last two decades—implications for sediment management. Water 12, 1965 (2020).

573  36. Bernardi Aubry, F., Acri, F., Finotto, S. & Pugnetti, A. Phytoplankton dynamics and water quality in the Venice lagoon.

574	Water 13, 2780 (2021).

575  37. Tognin, D. et al. Loss of geomorphic diversity in shallow tidal embayments promoted by storm-surge barriers. Sci. Adv. 8,

576	eabm8446, DOI: 10.1126/sciadv.abm8446 (2022).

577  38. Tosi, L., Da Lio, C., Cosma, M. & Donnici, S. Vulnerability of tidal morphologies to relative sea-level rise in the Venice

578	Lagoon. Sci. Total. Environ. 931, 173006 (2024).

579  39. Amos, C. L., Umgiesser, G., Ghezzo, M., Kassem, H. & Ferrarin, C. Sea surface temperature trends in Venice lagoon and

580	the adjacent waters. J. Coast. Res. 33, 385–395 (2017).

581  40. Sfriso, A., Wolf, M. A., Buosi, A., Sciuto, K. & Sfriso, A. A. Alien macroalgal rearrangement in the soft substrata of the

582	Venice lagoon (italy): impacts, threats, time and future trends. Sustainability 15, 8256 (2023).

583  41. Marchini, A., Ferrario, J., Sfriso, A. & Occhipinti-Ambrogi, A. Current status and trends of biological invasions in the

584	lagoon of Venice, a hotspot of marine nis introductions in the mediterranean sea. Biol. invasions 17, 2943–2962 (2015).

585  42. Sfriso, A., Buosi, A., Wolf, M. & Sfriso, A. Invasion of alien macroalgae in the Venice lagoon, a pest or a resource?

586	Aquatic Invasions 15, 245–270 (2020).

587  43. Piccardi, F. et al. Assessing the impact of the invasive ctenophore mnemiopsis leidyi on artisanal fisheries in the Venice

588	lagoon: an interdisciplinary approach. Hydrobiologia 1–19 (2024).

589  44. Stasolla, G., Tricarico, E. & Vilizzi, L. Risk screening of the potential invasiveness of non-native marine crustacean

590	decapods and barnacles in the mediterranean sea. Hydrobiologia 848, 1997–2009 (2021).

591  45. Bertocchi, D. & Visentin, F. “the overwhelmed city”: Physical and social over-capacities of global tourism in Venice.

592	Sustainability 11, 6937 (2019).

593  46. Cecchi, T. Analysis of volatiles organic compounds in Venice lagoon water reveals covid 19 lockdown impact on

594	microplastics and mass tourism related pollutants. Sci. Total. Environ. 783, 146951 (2021).

595  47. Walsh, C. et al. Trade and trade-offs: Shipping in changing climates. Mar. Policy 106, 103537 (2019).

596  48. Müller-Casseres, E., Edelenbosch, O. Y., Szklo, A., Schaeffer, R. & van Vuuren, D. P. Global futures of trade impacting

597	the challenge to decarbonize the international shipping sector. Energy 237, 121547 (2021).

598  49. Jaramillo, P. et al. Transport. In Shukla, P. R. et al. (eds.) Climate Change 2022: Mitigation of Climate Change.

599	Contribution of Working Group III to the Sixth Assessment Report of the Intergovernmental Panel on Climate Change,

600	1049–1160 (Cambridge University Press, Cambridge, United Kingdom and New York, NY, USA, 2022).

601  50. Hanson, S. E. & Nicholls, R. J. Demand for ports to 2050: Climate policy, growing trade and the impacts of sea-level rise.

602	Earth’s Futur. 8, e2020EF001543 (2020).

603 51. Haasnoot, M., Lawrence, J. & Magnan, A. K. Pathways to coastal retreat. Science 372, 1287–1290 (2021).

604  52. Nicholls, R. J. et al. Stabilization of global temperature at 1.5 c and 2.0 c: implications for coastal areas. Philos.

605	Transactions Royal Soc. A: Math. Phys. Eng. Sci. 376, 20160448 (2018).

606  53. Magnan, A. K. & Duvat, V. K. Towards adaptation pathways for atoll islands. insights from the maldives. Reg. Environ.

607	Chang. 20, 1–17 (2020).

608  54. Van Alphen, J., Haasnoot, M. & Diermanse, F. Uncertain accelerated sea-level rise, potential consequences, and adaptive

609	strategies in the netherlands. Water 14, 1527 (2022).

610  55. Kool, R., Lawrence, J., Larsen, M. A. D., Osborne, A. & Drews, M. Spatiotemporal aspects in coastal multi-risk climate

611	change decision-making: Wait, protect, or retreat? Ocean. & Coast. Manag. 258, 107385 (2024).

612  56. Lawrence, J. et al. Dynamic adaptive pathways planning for adaptation: lessons learned from a decade of practice in

613	aotearoa new zealand. J. Integr. Environ. Sci. 22, 2451424 (2025).

614  57. Cao, A. et al. Future of asian deltaic megacities under sea level rise and land subsidence: current adaptation pathways for

615	tokyo, jakarta, manila, and ho chi minh city. Curr. opinion environmental sustainability 50, 87–97 (2021).

616  58. Garner, G. G. et al. Ipcc ar6 sea level projections, DOI: 10.5281/zenodo.6382554 (2021).

617  59. Zanchettin, D., Bruni, S., Thiéblemont, R. & Rubinetti, S. Data from article "sea-level rise in Venice: historic and future

618	trends (review article)", DOI: 10.5281/zenodo.5139890 (2021).

619  60. Garner, G. et al. Ipcc ar6 wgi relative sea level projection distributions without background component, DOI: 10.26050/

620	WDCC/AR6.IPCC-DDC_AR6_Sup_DistBC (2023).

621  61. Kopp, R. E. et al. The framework for assessing changes to sea-level (facts) v1.0: a platform for characterizing parametric

622	and structural uncertainty in future global, relative, and extreme sea-level change. Geosci. Model. Dev. 16, 7461–7489,

623	DOI: 10.5194/gmd-16-7461-2023 (2023).

624 62. Tosi, L., Teatini, P. & Strozzi, T. Natural versus anthropogenic subsidence of Venice. Sci. reports 3, 2710 (2013).

625  63. Da Lio, C., Teatini, P., Strozzi, T. & Tosi, L. Understanding land subsidence in salt marshes of the Venice lagoon from sar

626	interferometry and ground-based investigations. Remote. sensing environment 205, 56–70 (2018).

627  64. Tosi, L., Teatini, P., Carbognin, L. & Brancolini, G. Using high resolution data to reveal depth-dependent mechanisms that

628	drive land subsidence: The Venice coast, italy. Tectonophysics 474, 271–284 (2009).

629  65. Calafat, F. M., Chambers, D. P. & Tsimplis, M. N. Mechanisms of decadal sea level variability in the eastern north atlantic

630	and the mediterranean sea. J. Geophys. Res. Ocean. 117 (2012).

631 66. Scarascia, L. & Lionello, P. Global and regional factors contributing to the past and future sea level rise in the northern

632	adriatic sea. Glob. planetary change 106, 51–63 (2013).

633  67. Tosi, L., Strozzi, T., Da Lio, C. & Teatini, P. Regional and local land subsidence at the Venice coastland by terrasar-x psi.

634	Proc. Int. Assoc. Hydrol. Sci. 372, 199–205 (2015).

635  68. Gambolati, G. & Teatini, P. Venice shall rise again: Engineered uplift of Venice through seawater injection (Elsevier,

636	London, 2013).

637  69. Lionello, P. et al. Extreme storm surges in the gulf of Venice: Present and future climate. In Flooding and Environmental

638	Challenges for Venice and Its Lagoon: State of Knowledge, 59–65 (Cambridge University Press, 2005).

639  70. Lionello, P. et al. Extremes floods of Venice: characteristics, dynamics, past and future evolution. Nat. Hazards Earth Syst.

640	Sci. Discuss. 2020, 1–34 (2020).

641  71. Conte, D. & Lionello, P. Characteristics of large positive and negative surges in the mediterranean sea and their attenuation

642	in future climate scenarios. Glob. Planet. Chang. 111, 159–173 (2013).

643  72. Lionello, P., Mufato, R. & Tomasin, A. Sensitivity of free and forced oscillations of the adriatic sea to sea level rise. Clim.

644	Res. 29, 23–39 (2005).

645  73. Aerts, J. C. et al. Pathways to resilience: adapting to sea level rise in los angeles. Annals New York Acad. Sci. 1427, 1–90,

646	DOI: https://doi.org/10.1111/nyas.13917 (2022). https://nyaspubs.onlinelibrary.wiley.com/doi/pdf/10.1111/nyas.13917.

647  74. Libertino, A., Ganora, D. & Claps, P. Evidence for increasing rainfall extremes remains elusive at large spatial scales: The

648	case of italy. Geophys. Res. Lett. 46, 7437–7446 (2019).

649  75. Parisi, S. G., Alimonti, G. & Mariani, L. Mean and extreme precipitation regime in north and central italy – between

650	stability and change. Italian J. Agrometeorology DOI: 10.36253/ijam-3315 (2025).

651  76. Faggian, P. & Trevisiol, A. Climate extreme scenarios affecting the italian energy system with a multi-hazard approach.

652	Bull. Atmospheric Sci. Technol. 5, 4 (2024).

653 77. Rinaldo, A. et al. Sea level rise, hydrologic runoff, and the flooding of venice. Water Resour. Res. 44 (2008).

654  78. Zuliani, A., Zaggia, L., Collavini, F. & Zonta, R. Freshwater discharge from the drainage basin to the Venice lagoon (italy).

655	Environ. Int. 31, 929–938 (2005).

656  79. Caporin, M. & Fontini, F. Damages evaluation, periodic floods, and local sea level rise: the case of Venice, italy. In

657	Handbook of environmental and sustainable finance, 93–110 (Elsevier, 2016).

658 80. Aerts, J. C. J. H. A review of cost estimates for flood adaptation. Water 10, DOI: 10.3390/w10111646 (2018).

659  81. Scrinzi, D., Ferrentino, R., Baù, E., Fiori, L. & Andreottola, G. Sewage sludge management at district level: reduction and

660	nutrients recovery via hydrothermal carbonization. Waste Biomass Valorization 14, 2505–2517 (2023).

661  82. von Sperling, M. & Salazar, B. L. Determination of capital costs for conventional sewerage systems (collection, transporta-

662	tion and treatment) in a developing country. J. water, sanitation hygiene for development 3, 365–374 (2013).

663 83. Somers Cocks, A. Venice erects glass barriers around st mark’s basilica to fight flooding (2022-10).

664  84. Manente, M. & Montaguti, F. L’impatto economico del turismo veneziano. Quaderni Insula 6, 29–37 (2004).

665  keywords

666  Venice, sea-level rise, climate change, adaptation pathways, sustainable development

667  Acknowledgements

668  The authors acknowledge Fondazione Eni Enrico Mattei (FEEM) for the organization of a workshop in February 2023

669  instrumental for this study.

670  Funding

671  CG acknowledges financial support from the Fondazione Eni Enrico Mattei (Adapt@Ve Programme). VDF was supported

672  by the European Union’s Horizon 2020 research and innovation programme through the Pathways2Resilience project (Grant

673  Agreement No. 101093942). UP, RJN, and ATV acknowledge financial support from the Horizon Europe project CoCliCo

674  (Coastal Climate Core Services).

## 675  Author contributions statement

676  PL: coordination, conceptualization, discussion, writing of the whole text, graphics of figures 3, 4, 5 and 6

677  VDF: conceptualization, discussion and description of measures and strategies and writing of the corresponding text

678  UP: computation of costs estimates and writing of the corresponding text

679  LT: assessment of the elevation of ground and defence systems, production of figures 1 and 2

680  GLC: conceptualization, discussion, analysis of sea-level rise

681  RJN: conceptualization, discussion, methodology for cost estimates, contribution to overall writing

682  PT conceptualization, discussion and detailed cost estimate of the measure for ground elevation

683  WC, RC, CG, JH, AS, ATV, GU : conceptualization and discussion

684  MH: conceptualization, discussion, graphics of figures 4,5,6

685

## 686  Additional information

687  Data Availability

688  Sea-level projection data used in this study are publicly available from the following repositories:

689  Zenodo: https://doi.org/10.5281/zenodo.6382554

690  Zenodo: https://doi.org/10.5281/zenodo.5139890

691  IPCC AR6 Data Distribution Centre (Distilled and Bias-Corrected Sea Level Projections):

692  https://doi.org/10.26050/WDCC/AR6.IPCC-DDC_AR6_Sup_DistBC

693  Code and associated data described in:

694  https://doi.org/10.5194/gmd-16-7461-2023 (see section “Code and Data Availability”).

695

696  Land elevation datasets of the areas surrounding the Venice Lagoon and of the lagoon islands used in this study are publicly

697  available from the Italian Ministry of the Environment and Energy Security via the National Geoportal

698  (https://gn.mase.gov.it/portale/en/home)

699  and the Spatial Data Access Portal

700  (https://sim.mase.gov.it/portalediaccesso/mappe/#/viewer/new).

701

702  For Figure 1 and related analyses, land elevation data were extracted from a Digital Terrain Model (DTM) provided by

703  the Veneto Region, available at

704  https://idt2.regione.veneto.it/idt/downloader/download.

705

706  High-resolution elevation datasets of the Venice lagoon boundaries with 1 m and 2 m ground resolution (Figure 2 and

707  associated text), derived from airborne LiDAR surveys, were acquired by the Italian Ministry of the Environment and the

708  Protection of Land and Sea under the Extraordinary Environmental Detection Plan (PST-A). These datasets are publicly

709  accessible via the portals of the Italian Ministry of the Environment and Energy Security listed above.

710

711  Ground elevation data of the Venice historical centre used in this study to compute the fraction of flooded area are pub-

712  licly available from the RAMSES project promoted by the Municipality of Venice and implemented by Insula S.p.A., via the

713  Insula S.p.A. portal (http://smu.insula.it/index.html).

714

715  Information used for cost estimates was obtained from the peer-reviewed scientific literature cited in the manuscript.

## 716 Competing interests

717  The authors declares no competing interest and no conflict of interest in relation to the work described in this manuscript