Table 1.  Stroke survivors’ and physiotherapists’ thoughts on Prototype One of the mirror therapy equipment and its set-up

| Aspects of the design | Stroke Survivors’ thoughts | Physiotherapists’ thoughts |
| --- | --- | --- |
| Mirror size | - The mirror needs to be half the size of Prototype One - Use a big box for the mirror | - The bigger mirror is better to see the foot - to be ¾ the size of Prototype One |
| Reflection of less paretic foot and shank | - 5-10 degree angle to see reflection without having a bent back - Another mirror placed in front of the less paretic foot - Para-scope above so that reflection can be adjusted | - 10-15 degree of inclination of the mirror to able to see the foot without bending the torso |
| The base for the mirror | - Hinge or spring load to connect mirror with base - preferably a one-piece base - Smaller, narrower base than Prototype One so that it can be slid under a chair | - Different surface; use texture to prevent foot sliding - Use rubber underneath to prevent sliding of the base on a laminate floor |
| Foot support to allow ankle movement; | - Need only one per side - Use pin, different slots or magnetic supporter to adjust it according to an individual’s height | - Better on a runner with brake - Screw above to adjust position according to patient height - One supporter on the side that is movable and adjustable - Different surfaces - hard, soft, wobble - A not slippery surface like Prototype One - Curve in the middle to support the ankle, like the one in the trainer shoes with soft padding |
| How to hide the more paretic side so user cannot see it. | - The sheet is a good idea and easy to use | - The sheet is a good idea. - Maybe big mirror to hide the weak side - Frame covered with fabric to obscure fully - Something like an umbrella frame to click for open/close - Use wireframe to fold up |
| Sitting posture, | - Prefer to sit upright with back support - Use regular dining chairs | - Use a regular chair with back support - Support for the more paretic leg to prevent external rotation |
| Storage and portability | - Handle on the mirror so that it is easy to carry - Lighter weight than Prototype One - Able to store it in the house easily | - Connect and click together for use with only one hand - One piece |
| Safety features | - Rounded corners - Plastic mirror - Lighter weight | - Rounded corners - Plastic mirror - Lighter weight and a smaller size |
| Thoughts from participants that were outside the pre-defined themes | - “We will be motivated to use the equipment because of the simplicity of the design” - Needed are clear instructions, e.g., where to place the foot. - It is important that the cost of the mirror therapy equipment is reasonable so that it can be used by most people - "I like the idea of including us in the design of the equipment, that's very nice." | - Some people may need support to prevent external rotation of the more paretic lower limb. |

Table 2.  Themes from participants about the second prototype of mirror therapy equipment and its set-up

| Aspects of the design | Stroke Survivors’ thoughts | Physiotherapists’ thoughts |
| --- | --- | --- |
| Overall design | - Approval of the overall design; “it’s much better than the last version”. - “I can see all my foot. Personally, it's perfect for me like this, that’s all you need for not leaning over.” | - Approval of the overall design. “That’s much better.” |
| Size | - Reduction of the size will be better. | - Still heavy, reduction in the size is needed. “Yeah that’s quite heavy.” |
| One piece | - Hinge to each other to make it connect. | - Can use it with one hand. |
| Angle | - Different angles of the mirror would be preferable to allow sight of the ankle and avoid having to bend the back. | - The mirror needs to be angled to allow a clear reflection, but with a certain limit to avoid a distorted image. |